Parts list for Power Supply (PRC-10 / RA-105)

| Item | Quantity | Reference | Description | ELFA part no. |
| --- | --- | --- | --- | --- |
| 1 | 6 | C1, C2, C3, C4, C16, C20 | 47nF | 65-736-38 |
| 2 | 7 | C5, C6, C11, C12, C15, C17, C18 | 1000uF 16V | 67-228-39 |
| 3 | 4 | C7, C8, C9, C10 | 100uF 100V | 67-231-83 |
| 4 | 2 | C13, C19 | 330uF 16V | 67-155-69 |
| 5 | 1 | C14 | 1uF 16V | 65-228-17 |
| 6 | 4 | D1, D2, D7, D8 | STPS340 | 70-288-48 |
| 7 | 4 | D3, D4, D5, D6,D12 | 11DQ10 | 70-221-06 |
| 8 | 6 | L1, L2, L3, L5, L6, L7 | 470uH | 58-868-09 |
| 9 | 1 | L4 | 47uH | 58-864-86 |
| 10 | 2 | R1, R3 | 1,54k | 4M |
| 11 | 1 | R2 | 17,8k | 4M |
| 12 | 1 | R4 | 22k | 4M |
| 13 | 1 | R5 | 300 | 4M |
| 14 | 1 | R6 | 2k | 4M |
| 15 | 1 | R7 | 750 | 4M |
| 16 | 1 | R8 | 220k | 4M |
| 17 | 1 | T1 - Core | ETD29 | 58-760-08 |
|  | 1 | T1 - Bobbin | “ | 58-760-16 |
|  | 2 | T1 - Clip | “ | 58-760-24 |
| 18 | 1 | U6 | LM2588T-adj | 73-280-57 |
| 19 | 1 | U8 | LM2575T-adj | 73-277-94 |
| 20 | 1 | Cooling bracket | 1mm Copper | Drawing #1 |
| 21 | 1 | PC board, double sided | Epoxy/glass | Drawing #2 |
| 22 | 1 | Zener Diode | P6KE30A | ? |

All resistors 1/4W metal film.

All capacitors in the uF-range should have low-esr.

Other capacitors should be ceramic.

Closer description of each part may be found on individual data sheets available from

ELFA  internet site:  http://www.elfa.se

Click on data sheet symbol for the part of interest.

# October 2005 – JE